WinCE 6.0 MCS98xx Driver Installation:

The following procedure explains how to install the MCS98xx PCI to  multiport serial and parallel  module driver under WinCE.

1. Obtain a copy of MCS98xx WinCE 6.0 driver package and extract it to your computer.

Copy the extracted “MCS98xx” folder to %WINCEROOT%\Osdesigns\ Osdesign1

(Example: C:\WINCE600\OSDesigns\OSDesign1\OSDesign1 or C:\WINCE600\OSDesigns\OSDesign1\OSDesign1).

2. Start WinCE Platform Builder, select File, and open Project.

3. Name the Project / Osdesigns, Click “OK” once done with.

4. Click “Next” to select “BSP”.

5. Now we have the BSP’s option to select for, Select “CEPC: X86” click “Next”.

6. Select “PDA Device” and Click “Next”

7. Select “Enterprise Web Pad” Click “Next”

8. Click “Next”

9. Select “LAN” and Select “Wired Local Area Network”, Click “Next”

10. Click “Finish”

11. Click “Acknowledge” for OBEX Server Warning.

Note: Make sure you select the Build for “Retail version”

12. From “Build” option select “Advanced Build Commands”, Select

“Build and Sysgen”

Note: Ensure that you get zero build errors, if not repeat the above steps.

13. Adding Drivers to built image.

a) In Solution Explorer view of workspace, Right Click the mouse by selecting “Subprojects” and select the option “Add existing Subproject”

*14. Copy the drivers (MCS9865Ce60) provided to you by “MosChip”

and Paste it in this folder.

15. Select the “MCS9865Ce60.pbpxml” and Click “Open”

16. Now the drivers are added to the built image.

17. Build the drivers, by selecting the added subproject and right click the mouse,

Select the option “Build” make sure you get zero errors and zero warnings.

18. Make image, by selecting “Build”->”Make Run Time Image”.

Ensure zero errors.

Note: It is advisable that once we add drivers better to repeat the step 12 once and then come to step 19.

19. And now we can put the image in the CEPC device and work

with hardware provided for MCS98xx device.

Note: If one has already the image built, then they can follow steps from 14 to 19.