Silicon Image SuSE 9.0 Series Driver Installation

(SATA 3124 Controller)

# OVERVIEW

This driver disk adds support for the SiI 3124 SATA controller. The installation requires special care since SuSE 9.0 series reboots automatically.

# CREATE FLOPPY before INSTALLATION

1) The easiest way to create a floppy containing the drivers is to us
   “dd” as follows:

dd if=file_name.img of=/dev/fd0  (In a Linux System)
      where file_name will change with the distribution used.

2) Configure RAID SET using the controller BIOS if you want RAID
   functionality

3) The following BIOS BOOT sequence is recommended => CD, Hard Disk,
   Floppy

# INSTALL THE SuSE DRIVER

## Phase 1

By default SuSE wants to boot from the Hard Disk.  At Boot Time manual intervention is required to choose Installation.  See figure 1.

For the 3124 series controller insert the floppy disk but DO NOT type F6. Follow the normal instructions.

FIGURE 1.

At some point the system will AUTOMATICALLY reboot.  Unfortunately if you do not catch the reboot the system will not work properly on the second reboot.  In any case on the second reboot you MUST repeat the same steps described above. What follows is what is different (see Phase 2).

Phase 2

On the second reboot you must choose “Boot Installed System” since the system software is already partially on the hard disk.                      .

FIGURE 2.

This time complete installing the software until you get to the “finish” screen or you get to login as shown in figure 3. Last, you still need to run one final script.  Get a console window as before and execute the following:

# mkdir /tmp/f ; mount /dev/fd0 /tmp/f

# cd /tmp/f; ./upgrade_driver.sh

("sh ./upgrade_driver.sh" if minimum install)

At this point you are done. You can now reboot the system for normal operation.

FIGURE 3.